🚦 הרצף:

- אדום – 1 שנייה

- אדום + צהוב – 1 שנייה

- ירוק – 1 שנייה

- ואז loop() מתחיל מהתחלה אוטומטית

## סיכום קצר

- Charlieplexing חוסך פינים

- INPUT = ניתוק (High-Z)

- הדלקה כפולה נעשית ע"י הבהוב מהיר

- הקוד מחקה רמזור אמיתי

Here is a clear, structured explanation of the code in English, step by step.

## Overall idea

We have three LEDs (Red, Yellow, Green) connected using Charlieplexing with three microcontroller pins.

Required sequence (each state = 1 second):

- Red

- Red + Yellow

- Green

- Repeat

Because Charlieplexing cannot turn on two LEDs at the same time electrically, we use fast multiplexing to make Red and Yellow appear on together.

## Pin definitions

const int P1 = 2;

const int P2 = 3;

const int P3 = 4;

These are three digital pins that can be:

- HIGH

- LOW

- INPUT (high-impedance / disconnected)

Charlieplexing relies on switching between these states.

## allHighZ() – safe off state

void allHighZ() {

pinMode(P1, INPUT);

pinMode(P2, INPUT);

pinMode(P3, INPUT);

}

What it does:

- Sets all pins to INPUT

- This creates a High-Z (high impedance) state

- Prevents unwanted current paths and ghost LEDs

➡️ Every LED activation starts from this clean state.

## Turning on the Red LED

void redOn() {

allHighZ();

pinMode(P1, OUTPUT);

pinMode(P2, OUTPUT);

digitalWrite(P1, HIGH);

digitalWrite(P2, LOW);

}

How it works:

- Red LED anode → P1

- Red LED cathode → P2

- P1 = HIGH, P2 = LOW → current flows → Red LED turns on

- P3 stays disconnected (INPUT)

## Turning on the Yellow LED

void yellowOn() {

allHighZ();

pinMode(P2, OUTPUT);

pinMode(P3, OUTPUT);

digitalWrite(P2, HIGH);

digitalWrite(P3, LOW);

}

Same principle:

- P2 = HIGH

- P3 = LOW

- Only the Yellow LED conducts current

## Turning on the Green LED

void greenOn() {

allHighZ();

pinMode(P3, OUTPUT);

pinMode(P1, OUTPUT);

digitalWrite(P3, HIGH);

digitalWrite(P1, LOW);

}

- P3 = HIGH

- P1 = LOW

- Green LED turns on

## Red + Yellow using multiplexing

void redAndYellowOneSecond() {

unsigned long start = millis();

while (millis() - start < 1000) {

redOn();

delay(5);

yellowOn();

delay(5);

}

}

What happens here:

- millis() gives the elapsed time since the MCU started

- The loop runs for 1 second

- Inside the loop:

  - Red LED is on for 5 ms

  - Yellow LED is on for 5 ms

Why it looks like both are on:

- The switching is much faster than human vision

- The eye integrates the light

- Both LEDs appear to be on simultaneously

This is time-multiplexing, commonly used in displays and traffic lights.

## setup()

void setup() {

allHighZ();

}

- Initializes the system

- Ensures all LEDs are off at power-up

## loop() – traffic light logic

void loop() {

redOn();

delay(1000);

redAndYellowOneSecond();

greenOn();

delay(1000);

}

Sequence:

- Red ON for 1 second

- Red + Yellow ON (multiplexed) for 1 second

- Green ON for 1 second

- loop() repeats automatically

## Summary

- Charlieplexing allows multiple LEDs with few pins

- INPUT mode = electrical disconnection (High-Z)

- Multiple LEDs are simulated using fast multiplexing

- The code implements a simple traffic light controller

Bottom of Form